Documentation for Control IQ

Relating to:			CIQProc.EXE

Date:				January11, 2014

Program Revision:		4.0

Subject			API – Socket Messages

CIQProc has a built in Application Programming Interface (API) that utilizes TCP socket communications on port 1724.  CIQProc is the “server” or “listener” at the IP address of the computer it resides on, or in the case of multiple IPs on the same computer, CIQProc listens and responds on all IPs.

The socket communication is handled by a “message”.  Each message is comprised of a series of fields separated by a comma (,) and is usually a stream of printable ASCII characters.

CIQProc is “listening” for incoming message and will respond if the message fits the criteria defined below.  Every valid message is responded to, even if there is not return data.  CIQProc never initiates a message.  Every message includes a header and message type.  If the message is to include data, it will follow the message type as in:  header, message type, Data1, Data2, Data3,…DataN

A “client” program is used to communicate to the server by sending the message and retrieving the response.  The header initiated by the client must always be “CIQG”.

Note that no part of the message, including the header or message type is case sensitive.  I.E.   CIQG = ciqg = CiqG   and   GTV = gtv = Gtv

A typical message sent is:		CIQG,GTV,OATemp

A typical response is:		CIQP,AWD,75.6 deg F

A word about aliases:

In most cases, the ‘alias’ is enough to use with the messages.  Aliases are one portion of a fully qualified COP (Class / Object / Property).  The alias is the “object” portion of the COP.  Most message types require the alias only, but in some cases it is necessary to use to fully qualified COP.  For example, one of the message types, GTV (Get The Value) when used with the alias alone will return the .txtValue property.  If the alias were “OATemp” the response will be something similar to “73.4 deg F”.  In other words, unless specified, the program assumes that the alias is a ‘point’ as opposed to a ‘schedule’ or ‘system’ variable, and that the default property is ‘txtValue’.  Hence CIQG,GTV,OATemp  is equivalent to   CIQG,GTV,AI.OATemp.txtValue.  However, any other property must be returned with the fully qualified COP.  Some Examples:

CIQG,GTV,AI.OATemp.Value         will return    73.421567

CIQG,GTV,AI.OATemp.txtReason   will return    Alarm

Also, for simplicity, “pnt” may be used generically for the class of any ‘point’ with regards to common properties for all points.  Hence:

AI.OATemp.Value   is equivalent  to  Pnt.OATemp.Value

However, if you want to retrieve something specific to a point type, then the actual class must be use.  For example:

AI.OATemp.HighAlarm   is NOT retrievable as Pnt.OATemp.HighAlarm

Note:  As of Version 4 the “Class” portion of the COP is not needed.  The Object portion of the COP (usually the Alias) will return the value pf the default Property (usually .txtValue) Please refer to “Control IQ v4 – Class Properties” for details of the COPs

Supported Message Types

Type		Description				Example

Afterhours and Tenant Message

AHSubmit	AfterHours Submit			CIQG,AHSubmit,TID,TCode,Hour,Cst

AHTID	AfterHours Tenant ID		CIQG,AHTID,TID,TCode

Alarm Messages

AA		Acknowledge Alarms		CIQG,AA,alias[,alias,alias…]

AAA		Acknowledge All Alarms		CIQG,AAA

AIA		Alias(s) in Alarm			CIQG,AIA

ASIC1 Specific Messages

COA		Clear Override Asic1 Points	CIQG,COAP

GAD		Get Asic1 Details (see GPD)	CIQG,GAD,Alias[,XML]

SOA		Set Override Asic1 Points		CIQG,SOA,value,expire,alias[,alias…]

Groups and Lists Messages

GGD		Get Group Details			CIQG,GGD,groupname[,XML]

GGDUID	Get Group Details y UID		CIQG,GGDUID,groupUID[,XML]

GRP		Get Groups				CIQG,GRP[,viewmask]

LIST		Get Groups				CIQG,LIST,[matching string]

LISTN		Get Groups				CIQG,LISTN,[matching string]

Point related Messages

COAP		Clear Override All Points		CIQG,COAP

COP		Clear Override Points		CIQG,COP,alias[,alias,alias…]

GPD		Get Point Details			CIQG,GPD,Alias[,XML]

GTV		Get The Value			CIQG,GTV,alias[,alias,alias…]

PMS		Set PMS State			CIQG,PMS,numericID,ON (or OFF)

RLP		Reload Point(s)			CIQG,RLP[,alias,alias,alias…]

SOP		Set Override Points			CIQG,SOP,value,expire,alias[,alias…]

Scheduling Messages

CNS		Clear Normal Schedule		CIQG,CNS,alias[,alias,alias..]

COAS		Clear Override All Schedules	CIQG,COAS

COS		Clear Override Schedules		CIQG,COS,alias[,alias,alias…]

GES		Get The Exception Schedule	CIQG,GES,alias

GNS		Get The Normal Schedule		CIQG,GNS,alias

RLS		Reload Schedules			CIQG,RLS

SES		Set Exception Schedule		CIQG,SES,alias,onttime,offtime,date..

SNS		Set Normal Schedule		CIQG,SNS,alias,ontime,offtime,0,1,1

SNSR		Set Normal Schedule Record 	CIQG,SNSR,alias,ont,offt…

SNSRU	Set Normal Sched Rec Update	CIQG,SNSRU,alias,ont,offt…

SOS		Set Override Schedules		CIQG,SOS,value,expire,alias[,alias…]

Setpoints Messages

COSP		Clear Override Setpoints		CIQG,COSP,alias1[,alias2,alias3…,]

SOSP		Set Override Setpoints		CIQG,SOP,value,expiration,alias1..

SSP		Set Setpoints 			CIQG,SSP,value,expire,alias[,alias…]

System Related Messages

ACK		Acknowledge. 			CIQP,ACK

AWD		ACK with Data			CIQP,AWD,data[,data,data…]

DF		Download File			CIQG,DF,1724[,filename]

EVENT	Process Events			CIQG,EVENT,[EventName]

HOW		How Are You				CIQG,HOW

IAM		I Am					CIQP,IAM,node,version

RELOAD	Stop the processor and reload it	CIQG,Reload,1724

RSS		Reset System Status		CIQG,RSS

STS		Subsystem Sync Time		CIQG,STS

UF		Upload File				CIQG,UF,1724,filename

UPPREF	Update Preferences			CIQG,UPPREF,key,value

WDOG	Watch Dog (from SAI)		CIQG,WDOG

WHATSUP  	Test for activity			CIQG,WhatsUp

WHO		Who Are You				CIQG,WHO

User Messages

GVID  	Get Voice ID				CIQG,GRP,voiceusername

LOGIN	Get Login Credentials		CIQG,LOGIN,Username,Password

AA		Acknowledge Alarm

Format:	CIQG,AA,alias1[,alias2,alias3…,alias-n]

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

Acknowledge the alarm of the associated aliases.  If the alias is in alarm, it sets the AlarmState to 3.  If the alias is not in alarm, it takes no action.

See Also:	AAA, AIA

AAA		Acknowledge All Alarms

Format:	CIQG,AAA

Response:	CIQP,ACK

Acknowledge the alarm on all aliases that are in alarm.  If the alias is in alarm, it sets the AlarmState to 3.  If the alias is not in alarm, it takes no action.

See Also:  	ACK, AIA

ACK		Acknowledge

Format:	CIQG,ACK

Response:	CIQP,ACK

The acknowledge command is not normally sent by the client, in that it is part of the response message for other message types.  However, for completeness to the API, a received ACK message type will return an ACK response.  An ACK response has no other data.

See Also:	NAK

AIA		Aliases In Alarm

Format:	CIQG,AIA

Response:	CIQP,AWD,(alias1),(alias1.txtValue),(alias1.alarmstate),(alias2…

This will respond with all aliases that are in alarm, their associated .txtvalue property, and the associated .alarmstate value.  The message contains each of these three variable for each alias in alarm, separate by commas.  An example would be:

CIQP,AWD,OATemp,92 deg F,1,OAHumidity,99 %,1

See Also:	AA, AAA

AHSubmit	AfterHours Submit

Format:	CIQG,AHSubmit,TenantUID,TenantCode,Hours,Cost

Response:	CIQP,ACK

This will update the tenantstransactions table with the AH transaction

See Also:	AHTID

AHTID	AfterHours Tenant Details

Format:	CIQG,AHTID,TenantID,TenantCode

Response:	CIQP,ACK,TUID,TCUID,DurationLimit,HourlyCost,UserLevel

See Also:	AHSubmit

AWD		Acknowledge With Data

This command is the message type for many of the response message.  A NAL response has data that follows.

CNS		Clear Normal Schedule

Format:	CIQG,CNS,alias1 [,alias2,alias3…,alias-n]

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

Delete the occurrences of schedules with the name “alias”

COA		Clear Override ASIC/1

Format:	CIQG,COA,alias1 [,alias2,alias3…,alias-n]

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

Sets the ASIC/1s in auto mode (if state overridden)

See Also:	SOA

COAP		Clear Override on All Points

Format:	CIQG,COAP

Response:	CIQP,ACK

Any ‘point’ that has been overridden will be set back to automatic mode. I.E. CIQProc will make the decision regarding the state or value of the point.

See Also:	COP, SOP

COAS		Clear Override on All Schedules

Format:	CIQG,COAS

Response:	CIQP,ACK

Similar to COAP, this will remove the override from any ‘schedule’ alias and set it back in automatic mode. I.E. CIQProc will make the decision regarding the state or value of the schedule.

See Also:	COS, SOS

COP		Clear Override Points

Format:	CIQG,COP,alias1 [,alias2,alias3…,alias-n]

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

Similar to COAP, this will remove the override from any ‘point’ alias provided as part of the message and set it back in automatic mode. I.E. CIQProc will make the decision regarding the state or value of the point.

See Also:	COAP, SOP

COS		Clear Override Schedules

Format:	CIQG,COS,alias1 [,alias2,alias3…,alias-n]

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

Similar to COAS, this will remove the override from any ‘schedule’ alias provided as part of the message and set it back in automatic mode. I.E. CIQProc will make the decision regarding the state or value of the schedule.

See Also:	COAS, SOS

COSP		Clear Override Setpoint

Format:	CIQG,COSP,alias1 [,alias2,alias3…,alias-n]

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

Similar to COP, this will remove the override from any ‘setpoint’ provided as part of the message and set it back in automatic mode.

See Also:	SOSP

DF		Download File

Format:	CIQG,DF,xxxx [,filename]

CIQG,DF,xxxx

Response:	CIQP,ACK

If the file is not found the response is  CIQP,NAK

This will transfer a file from SAI’s servers and place it in the directory structure that is defined as part of the filename.  For example of the file name were c:\myfile.txt, then the program would download myfile.txt from the ftp server and place it in the c:\ directory on the local machine.  If the filename is left blank, it assumes the filename is CIQProc.EXE so it downloads the latest version. 1724 is the “authority” to allow a download to happen.  If the authority is 2724, then the latest CIQProc.exe will be downloaded, CIQProc will be stopped and restarted.

See Also:	UF, RELOAD

xxxx is provided by SAI as a password to access the data.

EVENT	Process Events

Format:	CIQG,EVENT,[EventName]

Response:	CIQP,ACK

If the Event is not found the response is  CIQP,NAK

This process an Event as defined in the Events table of the database.  The event can be a series of SQL commands against the database, Socket API calls (as defined in this document) and Email addresses (to send the status of the Event process.

GAD		Get ASIC/1 Details

This command is still supported for legacy systems, but has been replaced with GPD.

GES		Get Exception Schedule

Format:	CIQG,GES,alias

Response:	CIQP,AWD,startingdate,numberofdays,ontime,offtime

See Also:	GNS, CNS, SNS, SES

GGD		Get Group Details by Name

Format:	CIQG,GGD,groupname[,XML]

Response:	CIQP,AWD,p1.Name,P1.txtValue,P1.txtReason[,p2.name…

Alternate response is XML Stream

See Also:	GGDUID, GRP

GGDUID	Get Group Details by Group UID

Format:	CIQG,GGDUID,groupUID[,XML]

Response:	CIQP,AWD,p1.Name,P1.txtValue,P1.txtReason[,p2.name…

Alternate response is XML Stream

See Also:	GGD, GRP

GNS		Get Normal Schedule

Format:	CIQG,GNS,alias

Response:	CIQP,AWD,ontime,offtime,s,m,t,w,r,f,s,h

See Also:	GES, CNS, SNS, SES

GPD		Get Point Details

Format:	CIQG,GPD,alias[,,XML]

Response:	CIQP,AWD,…

Alternate response is XML Stream

This returns a set of Key-Value pairs, comma separated for all of the      pertinent details for the specific alias.

See Also:	GAD

GRP		Get Point Details

Format:	CIQG,GRP[,viewmask][,XML]

Alternate response is XML Stream

Response:	CIQP,AWD,Grp1UID,GRP1Name,GRP2UID,GRP2Name..

Alternate response is XML Stream

If the viewmask is omitted (or set to 0) all groups are returned.  If viewmask > 0 only those groups that match the viewmask are returned.

See Also:	GGD, GGDUID

GTV		Get The Value

Format:	CIQG,GTV,alias1 [,alias2, alias3….,alias –n]

Response:	CIQP,AWD,data1 [,data2,data3….,data-n]

If one of the aliases is not found the response is  CIQP,NAK

This is the primary message type in the socket communications API.  It allows retrieving that value of any point, schedule, or system variable as well including most of the properties.  If the “alias” is defined as the alias alone I.E.  OATemp, the API will assume it to be a “point” class (COP) and will return the .txtValue property. From the discussions above, “Pnt” can be substituted for any point class for simplicity.   Hence the following tree lines return the exact same data:

CIQG,GTV,OATemp

CIQG,GTV,AI.OATemp,txtValue

CIQG.GTV.Pnt.OATemp.txtValue

If the alias type, is not a ‘point’ or the property is not ‘.txtValue’ then the fully qualified COP must be entered.  Some examples of the fully qualified COPs with there simulated results would be:

Pnt.OATemp.txtValue		72.5 deg F

Pnt.OATemp.value			72.47843

Pnt.OATemp.Reason			16

Pnt.OaTemp.txtReason		Normal

Sched.Building.txtValue		On

Sched.Buidling.txtReason		TOD	(Time of Day)

Sys.System.SiteName			New York Hilton

Sys.System.SocketAddress		198.123.75.45

Sys.System.Version			v2.0.1.3

Sys.System.UpSince			7/24/2004  15:31:21

A complete list of available properties is define in other documents.

See Also:	SOP, SOS, COP, COAP, COS, COAS, AA, AAA

GVID		Get Normal Schedule

Format:	CIQG,GRP,voiceusername

Response:	CIQP,AWD,spokenname,accesslevel

This returns the level of the voice user to CIQVoice.

HOW		How Are You

Format:	CIQG,HOW

Response:	CIQP,AWD,(scan time)

This is a very simple message type that returns the amount of time that CIQProc is taking to make a complete scan of all process and all subsystems.

See Also:	WHO, WDOG, RLS

IAM		I Am

‘Iam’ is the response message for the WHO message type.

See:		WHO

LIST		List of Aliases

Format:	CIQG,LIST,[matching string]   (like clause)

Response:	CIQP,AWD,Alias1,Alias2,Alias3….

This returns a list of all aliases in the points table that match the matching string criteria.  If the matching string is omitted all aliases are returned.

See Also:	LISTN

LISTN		List of Aliases and Names

Format:	CIQG,LISTN,[matching string]   (like clause)

Response:	CIQP,AWD,Alias1,Name1,Alias2,Name2,Alias3,Name3….

This returns a list of all aliases and the associated point names in the points table that match the matching string criteria.  If the matching string is omitted all aliases are returned.

See Also:	LIST

LOGIN	Login a User

Format:	CIQG,LOGIN,Username,Password

Response:	CIQP,AWD,Description,ViewMask,EditMask,Description

This the user credentials to control what a user has access to do and see.

PMS		Sets the state of a room based on the NumericID

Format:	CIQG,PMS,numericID,ON (or OFF)

Response:	CIQP,AWD

This comes from CIQ-SA Interface to trigger the room change.

RELOAD	Stop the processor and reload it

Format:	CIQG,Reload,xxxx

Response:	GIQP,ACK

This tells the CIQProc program to reload itself, essentially stopping and starting the program.  The ACK, may actually not be sent if the process reloads fast enough.  This is the only exception that there may not be a response.  The 1724 is the ‘authorization’ for it to proceed.

Reload is required under the following conditions.

Changes to the subsystem(s) regarding

Alias name

Port Settings

Changes to the point(s) regarding:

Subsystem Type

Point Type

Address

New Point Added

Exiting Point Deleted

Existing Point Activated

Existing Point Deactivated

See Also:	DF, RLP, RLS

xxxx is provided by SAI as a password

RLP		Reload Points

Format:	CIQG,RLP[,alias,alias,alias…]

Response:	GIQP,ACK

If any alias is not found the response is  CIQP,NAK

This reloads the points and classes tables from the database in to CIQProc.  Once the database is altered (from a GUI program) for any parameter of a point(s), this message type needs to be sent to re-evaluate them.

If alias is empty, then all points are re-read from the database.  Note that if any part if the subsystem type, point type, or address fields are altered in a point(s) definition, then a “ReLoad” message type must be sent.  If a reload is sent then RLP is redundant.

See Also:	ReLoad

RLS		Reload Schedules

Format:	CIQG,RLS

Response:	GIQP,ACK

This refreshes all of the schedules from the database into CIQProc.  Once the database is altered (from a GUI program) for either standard or exception schedules, this message type needs to be sent to activate them

See Also:	COS, COAS, SOS

RSS		Reset System Status

Format:	CIQG,RSS

Response:	CIQP,ACK

This resets Sys.System.Status to ImOK and Sys.System.StatusTest= ‘’

Added in v2.0.1.11

See Also  HOW

SES		Set Exception Schedule

Format:	CIQG,SES,alias,ontime,offtime,date,days

ontime is the time for the schedule to come on in military time

Note that no colons are used, just four digits

0600

offtime is the time for the schedule to turn off in military format

Note that no colons are used, just four digits

2300

date is the starting date for the exception to take affect

mm-dd-yyyy

days is the number of days for it to be affective

1-99

Response:	CIQP,ACK   if it everything is ok

CIQP,NAK   if the data or alias is invalid

This sets the normal schedule relating to the alias.  Note that a RLS must be sent once to refresh the schedule list after all of the schedules are updated.

See Also:	SNS, RLS

SNS		Set Normal Schedule – Depricated use SNSR

Format:	CIQG,SNS,alias,ontime,offtime,S,M,T,W,R,F,S,H

ontime is the time for the schedule to come on in military time

Note that no colons are used, just four digits

0600

offtime is the time for the schedule to turn off in military format

Note that no colons are used, just four digits

2300

S,M,T,W,R,F,S,H is a 1 or 0 for assigning this schedule to what days

of the week.  Example would be 0,1,1,1,1,1,0,0 would assign

the schedule to Monday through Friday, not Sundays

and not Saturdays or Holidays.

Response:	CIQP,ACK   if it everything is ok

CIQP,NAK   if the data or alias is invalid

This creates a Normal schedule relating to the alias.  Note that a RLS must be sent once to refresh the schedule list after all of the schedules are updated. This creates a record per day of week

See Also:	SES, RLS, SNSR

SNSR		Insert a Normal Schedule Record

Format:	CIQG,SNSR,alias,ontime,offtime,S,M,T,W,R,F,S,H

ontime is the time for the schedule to come on in military time

Note that no colons are used, just four digits

0600

offtime is the time for the schedule to turn off in military format

Note that no colons are used, just four digits

2300

S,M,T,W,R,F,S,H is a 1 or 0 for assigning this schedule to what days

of the week.  Example would be 0,1,1,1,1,1,0,0 would assign

the schedule to Monday through Friday, not Sundays

and not Saturdays or Holidays.

Response:	CIQP,ACK   if it everything is ok

CIQP,NAK   if the data or alias is invalid

This creates a normal schedule relating to the alias.  Note that a RLS must be sent once to refresh the schedule list after all of the schedules are updated.  This creates a single record.

See Also:	SES, RLS, SNS

SNSRU	Update a Normal Schedule Record

Format:	CIQG,SNSRU,alias,ontime,offtime,S,M,T,W,R,F,S,H

ontime is the time for the schedule to come on in military time

Note that no colons are used, just four digits

0600

offtime is the time for the schedule to turn off in military format

Note that no colons are used, just four digits

2300

S,M,T,W,R,F,S,H is a 1 or 0 for assigning this schedule to what days

of the week.  Example would be 0,1,1,1,1,1,0,0 would assign

the schedule to Monday through Friday, not Sundays

and not Saturdays or Holidays.

Response:	CIQP,ACK   if it everything is ok

CIQP,NAK   if the data or alias is invalid

This updates a normal schedule relating to the alias.  Note that a RLS must be sent once to refresh the schedule list after all of the schedules are updated.  This creates a single record.

See Also:	SES, RLS, SNS

SOA		Set Override ASIC/1 Point

Format:	CIQG,SOA,value,experiation,username=name

value – the value to override the alias(s) to.

expiration – three possible formats.

leave blank for no expiration

CIQG,50,,OADamper,EADamper

Date/Time for a specific time to expire in the future

Format is MM-DD-YYYY [hh:mm]

CIQG,1,7-31-2004 13:30,FanStart

Duration in Hours (from the time the command is 
			   	     issued)

Format   xxx.xx hours

CIQG,100,1.25,VAV101

(override the valve for 1hour and 15 minutes)

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

See Also:	COP, COAP, COA

SOP		Set Override Points

Format:	CIQG,SOP,value,expiration,alias1 [,alias2,…..alias-n]

value – the value to override the alias(s) to.  If this is a digital   point

use 1 for On, and 0 for Off.

expiration – three possible formats.

leave blank for no expiration

CIQG,50,,OADamper,EADamper

Date/Time for a specific time to expire in the future

Format is MM-DD-YYYY [hh:mm]

CIQG,1,7-31-2004 13:30,FanStart

Duration in Hours (from the time the command is

issued)

Format   xxx.xx hours

CIQG,100,1.25,CHValve

(override the valve for 1hour and 15 minutes)

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

This will override or ‘set’ a point to a specific value until the program passes the expiration time (assuming there IS an expiration time).  Multiple aliases may be specified, but each will share the same value and the same duration,

See Also:	COP, COAP, COA, SOA

SOS		Set Override Schedules

Format:	CIQG,SOS,value,expiration,alias1 [,alias2,…..alias-n]

value – the value to override the alias(s) to.  Use a 1 for On, and 0 for Off.

expiration – three possible formats.

leave blank for no expiration

CIQG,1,,AH1Sched,AH2Sched

Date/Time for a specific time to expire in the future

Format is MM-DD-YYYY [hh:mm]

CIQG,1,7-31-2004 13:30,FanSched

Duration in Hours (from the time the command is

issued)

Format   xxx.xx hours

CIQG,0,1.25,Lights

(override the valve for 1hour and 15 minutes)

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

This will override or ‘set’ a schedule on or off depending on the value until the program passes the expiration time (assuming there IS an expiration time).  Multiple aliases may be specified, but each will share the same value and the same duration,

See Also:	COS, COAS

SOSP		Set Override Set Points

Format:	CIQG,SOP,value,expiration,alias1 [,alias2,…..alias-n]

value – the value to override the setpoint value.

expiration – three possible formats.

leave blank for no expiration

CIQG,50,,OADamper,EADamper

Date/Time for a specific time to expire in the future

Format is MM-DD-YYYY [hh:mm]

CIQG,1,7-31-2004 13:30,FanStart

Duration in Hours (from the time the command is

issued)

Format   xxx.xx hours

CIQG,100,1.25,CHValve

(override the valve for 1hour and 15 minutes)

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

This will override or ‘set’ a setpoint to a specific value until the program passes the expiration time (assuming there IS an expiration time).  Multiple aliases may be specified, but each will share the same value and the same duration,

See Also:	COSP, SSP

SSP		Set the Set Points (not overridden)

Format:	CIQG,SOP,value,alias1 [,alias2,…..alias-n]

value – the value to set the setpoint value.

Response:	CIQP,ACK

If any alias is not found the response is  CIQP,NAK

This will adjust a setpoint to a specific value.  Multiple aliases may be specified, but each will share the same value and the same duration,

See Also:	COSP

STS		Subsystem Time Sync

Format:	CIQG,STS

Response:	CIQP,ACK

This forces all subsystem to sync their time immediately.

UF		Upload File

Format:	CIQG,UF,xxxx,filename

Response:	CIQP,ACK

If the file is not found the response is  CIQP,NAK

This will transfer a file from the local computer to SAI’s servers and place in the directory structure that is defined by sys.system.sitename.  For example if the file name were CIQPErr.txt, then the program would create a directory on SAI’s server (if it doesn’t exist) that is name after the site name of the local, then place the file in that directory. 1724 is the “authority” to allow a download to happen.

xxxx is provided by SAI as a password

See Also:	DF

UPPREF	Update Preferences Table

Format:	CIQG,UPPREF,key,value

Response:	CIQP,ACK

If the file is not found the response is  CIQP,NAK

This will insert a new record in the preference table (our update if it exists now).  Note:  A RELOAD may need to be executed if this take immediate action from Proc.

WDOG	Watch Dog (from SAI)

Format:	CIQG,WDOG

Response	CIQG,ACK

If the INI files is set for “ExpectWatchDog=True” then this message type needs to be sent to CIQProc on some frequency (once per hour).  If CIQProc, does NOT receive this pulse within a 2 hour time frame, it will notify users based on variables in the alarm_processes table with an alias of “WatchDog”

See Also:

WhatsUp

Format:	CIQG,WhatsUp

Response	CIQG,IAm

This is just a simple response that indicates that the main processor (CIQProc) is alive.  It is a good way to start out a socket communication since the response is always the same.

See Also:

WHO		Who Are You

Format:	CIQG,WHO

Response:	CIQG,IAM,node,version

Node will contain the ‘name’ of the processing node (normally ‘ControlIQ’) and version will contain sys.system.version (similar to ‘v2.0.1.3’)

See Also:	HOW