Control IQ – Forecast Weather Driver Details

CIQProc version:	5.0.0.0

Date:			April 20th, 2019

This document contains the details for using the Forecast Driver

Personality field details in the Subsystem table

Subsystem Type:		ForecastIO

Subsystem Settings:		None

Personality field details in the Subsystem table

Latitude=41.23

Longitude=-87.31

Multiple subsystems can be active on the same version of Proc.

Point Details:

Address Schema:		[Property.]KeyWord.x

Where

Property 	If provide must match the PropertyID in the Properties Table.  Latitude

and longitude are read from that table.

KeyWord	is one of the following:

Min*		- °F  Minimum Temperature

Max*		- °F  Maximum Temperature

Tem*		- °F  Outside Air Temperature (or OAT)

Dew*		- °F  Dew Point Temperature (or DP*)

Ap* 		- °F  Apparent Temperature (or WindChill)

Hum*		- %rh Outside Air Humidity (or OAH or RH)

POP*		- %   Probability of Perciptation

Rain		- %	  Chance of Rain Showers

Snow		- %	  Change of Snow Showers

Sky			- %	  Cloud Cover (or Cloud*)

wDir		- °	  Wind Direction

WSpd		- mph Wind Speed

Sunrise	- Sunrise Time at the location	   (HHmm)

Sunset		- Sunset Time at the location	   (HHmm)

Time		- Current Time at the location    (HHmm)

Wet*		- Wetbulb Temperature (or WB)

Ent*		- Enthalpy of outside air

x		Is the number of hours ahead to forecast (0 = now)

Range is in hours from 0 to 120 (5 days)

Example	OAT.0		- Is the present Outside Air Temperature

OAT.24	- Is the forecasted OAT in 24 hours

OAT.48	- Is the forecasted OAT in 48 hours

The * above is for a wildcard

Max.0, MaxTemp.0, MaxT.0, Maximum.0 will all return the same data.